A Limit on the Performance of Distributed Systems

Erik Dirkx

Vrije Universiteit Brussel

Dept INFO

Pleinlaan  2

B-1050  Brussel

Belgium

Email : erik@info.vub.ac.be

# Internet : http://infoweb.vub.ac.be

### Abstract

A common and cost-effective way of improving the performance of systems is to construct them out of multiple processing elements.  With modern production technologies this is often even the only economically viable solution.

With execution time as the measure of performance it is shown that for every distributed system, there exists a number of processing elements for which a minimal execution time is obtained.  For a given technology and problem there is no point in dedicating more processing elements than this critical number to it with the aim of improving performance.  Systems exhibiting this behavior are for example VLSI chips, parallel and distributed computer systems, (tele)communication and computer networks.

### Introduction

The performance of distributed systems can be expressed in different ways.  In every case, a balance has to be found between the perspective of the “customer” and the “provider”. The former’s aim is to obtain a maximum amount of “utility” from a given amount of resources (cost).  The latter’s aim is to provide a certain amount of “utility” (revenue) with a minimal amount of resources

In information systems resources are memories, processing elements and interconnect.  Memories store information, “processing elements” transform information and “interconnect” transfers pieces of information from their storage or production place(s) to where they are needed.  The goal of the service “provider” is to maximize the utilization or throughput of resources the “system” is built from. This is typically done by time multiplexing some resources.  Typically this implies delay in the service to individual “customers”.

The viewpoint in this paper is the “customer”’s : utility is measured as service time : the user wants to obtain a result within a given period of time and to obtain this goal is willing to invest in more resources.

This formulation implies that the optimization of the “provider” i.e. increasing utilization at the cost of delay (by time sharing resources) decreases the utility of the “customer”.  Only for simplicity, resources are considered to be instances of a generic “processing element” that includes storage, computational and interconnect capabilities.

### Speedup

The model for a customer’s behavior is that a minimal amount of service is received for a “sequential” version on a single “processing element” : Ts.

By increasing the amount of resources, i.e. moving to a distributed or parallel implementation, service is improved : Tp.

To be independent of technology and architecture, the benefit to a “customer” is measured in a relative way as “speed-up” :

σ=TsTp

Intuitively, speed-up increases with the amount of resources dedicated to the job, i.e. when the time to finish a job decreases.

Tp can be decomposed into two factors :

Tp=Tpcomp+Tpcomm

Where Tpcomp is the “compute” time and Tpcomm is the “communication” time spent by the distributed “computation” or algorithm.  Typically by increasing the amount of resources compute time decreases.  Although the results are valid for other (less optimal) cases, for simplicity the optimal case is assumed : “compute” time decreases proportionally with the number of “processing elements”

Tpcomp=TsN

With N the number of “processing elements” (resources) used. Although N is clearly an integer variable, for simplicity all functions are assumed to be continuous functions of N.

The negative side of distributing a problem over multiple processing elements is the introduction of communication overhead : Tpcomm.  The critical point to observe is that

Tpcomm=f(N)

dfdN≥0With

Time spent in communications increases with the number of processing elements.  Although this condition could be invalid locally, e.g. due to the dynamic behavior of an interconnect, it is true on a global scale. Tpcomm is the global overhead : it is assumed to be proportional to the diameter of the interconnect.  It is clear that this value strongly depends on the application domain and the technology.

A proof for this claim can be made based on Moore’ s law :

In an interconnection network with a fixed degree for every node, the diameter must increase with the number of nodes.

Moore’s law can be written in the following way :

Assume a network has diameter 2k.  Although this is not necessary, assume for simplicity a network with in-degree equal to out-degree and constant : d.  The maximum number of nodes N in this network is given by

N=1+d+d(d−1)+d(d−1)2+…+d(d−1)k−1=O(dk)

Or written in a different way : k=Ω(logdN)*		Q.E.D.

Due to the fact that a communication link must consume some strictly positive amount of time, f(N) is a non decreasing function of N.  As in every real system d has to be finite, we even have :

dfdN>0

Subsitution of Tpcomp and Tpcomm in σ gives

σ=TsTsN+f(N)

There must exist at least one maximum for this expression because

dσdN=−11N+f(N)Ts2−1N2+1TsdfdN

1N2is a strictly decreasing function of N.   f(N) is a strictly increasing function whose derivative is at least proportional to 1N.  Hence there is at least one extremum (maximum) of σ(N) as a function of N as dσdN has at least one zero.

Beyond this extremum, speed-up decreases as a function of N, i.e. from a performance point of view, it is useless to add more processing resources …

A graphical representation of Tpcomp, Tpcomm and σ is given in the figure below.

## Conclusion

Based on very general assumptions, independent of technology and architecture, it has been shown that the acceleration of a given problem that can be obtained by distributing it on multiple “processing elements” is fundamentally limited.  After reaching (or in actual implementations probably in neighborhood of) the critical number of processing elements, other strategies to improve the performance of the distributed system have to be pursued.

 Permission to make copy of all or part of this material without fee is granted provided title of the publication and the author's name and address is given.

   f(x) is O(g(x)) iff  there are constants x0 and k such that for x > x0 : f(x)<  k.g(x)

f(x) is (g(x)) iff  there are constants x0 and k such that for x > x0 : f(x) > k.g(x)